Konica Minolta Maxxum 7D Digital Single Lens Reflex (SLR) Camera

Specifications

| Camera Type | Interchangeable-lens digital SLR camera |
| --- | --- |
| Lens Mount | Minolta A-type bayonet mount |
| Image Capture | Total pixels: 6.3 million, Effective pixels: 6.1 million CCD size: 23.5mm x 15.7mm, Interline primary color, interlace scan Sensitivity: Auto and 100, 200, 400, 800, 1600,and 3200 ISO equivalents Aspect ratio: 3:2 |
| Recording | Recording Media: TypeⅠand TypeⅡ CompactFlash Cards, Microdrive, SD Memory Cards and MultiMedia Cards with optional CompactFlash Adapter SD-SF File format: JPEG, RAW, RAW+JPEG (DCF 2.0 compliant, DPOF (supported by printing functions in version 1.1), Exif 2.2) Storage capacity: Approximate storage capacity of a 256MB: 3008x2000; 2256x1496; 1504x1000. RAW; 26 frames; -; -. RAW+JPEG; 19 frames; 21 frames; 23 frames. Extra fine; 41 frames; 72 frames; 157 frames. Fine; 81 frames; 141 frames; 292 frames. Standard; 138 frames; 235 frames; 463 frames. Color mode: Natural: sRGB, Natural +: sRGB, Adobe RGB Image quality mode: Standard, Fine, Extra-fine, RAW, RAW+JPEG (Fine) Image correction: Contrast, Color saturation, Sharpness, Hue (5 levels) Noise reduction: Available (at shutter speed longer than 1 second, ON/OFF selectable) Delete function: Single frame, Marked frames or All frames in a folder in card can be deleted. |
| White Balance Control | Automatic, Preset (Daylight, Shade, Cloudy, Tungsten, Fluorescent, Flash), Custom, Color temperature setting available |
| Playback | LCD monitor: 2.5-inch TFT color Total pixels: 207, 000 Navigation system: (Enlarged display, Automatic rotate function) Number of frame/s displayed: 1, Index (4, 9, 16 selectable) Display mode: Image only, Image + text, Image + text + histogram Enlarged playback (Up to approximately 5x), Luminance limit Display, File browser, Slideshow, Instant bracket playback |
| Autofocus System | Type: Through-the-lens (TTL) phase-detection system Sensor: CCD line sensors (9 points, 8 lines with center cross-hair sensor) Sensitivity range: EV-1 – EV18 (ISO 100) Autofocus area: Wide focus area, any local focus area is selectable from 9 point sensors. Focus mode: Autofocus/Manual focus can be selected. Autofocus mode: Single-shot AF, Continuous AF, Automatic Autofocus (DMF is selectable in menu). Predictive focus control for moving subjects. Auto-tracking focus point display Focus lock: Available (by pressing the shutter release button halfway down) AF illuminator: Available (with pre-flash) 3.3-16.4 ft. (1.0-5.0m) |
| Autoexposure System | TTL metering using 14-segment honeycomb-pattern SPC (14-segment Honeycomb-pattern/Center-weighted/Spot metering) Metering range: EV 0 - 20 (EV 3 - 20 in spot metering) * at ISO100 with f/1.4 lens Exposure modes: P/Full-auto program (Programmed AE with program shift), A (Aperture priority), S (Shutter priority), M (Manual) Exposure-compensation compensation: +/– 3 EV in 1/2 increments +/– 2 EV in 1/3 increments AE lock: Automatically activated with AF lock, Available with AE lock button |
| Flash | Flash metering: ADI, Pre-flash TTL, Manual flash control Guide number: 12 (in meters at ISO 100), 17 (in meters ISO 200) Coverage: 24mm Recycling time: Approximately 3 seconds Control: Manual switchover. Lift-up for Fill Flash, push down for Flash Cancel. Flash modes: Fill Flash, Flash Cancel, Red-eye Reduction (pre-flash), Wireless/Remote Off-camera Flash, Rear Flash Sync., High Speed Sync. Flash compensation: +/– 2 EV in 1/2 stop increments |
| External-flashes | Program Flash 5600HS(D), 3600HS(D), 2500D, Macro Ring Flash 1200* with flash sync. terminal * Macro Flash Controller is necessary. |
| Shutter | Type: Electronically-controlled, vertical-traverse, focal-plane Shutter Range: 30-1/4000 second, Bulb Flash sync speed: 1/160 second (with Anti-shake OFF), 1/125 second (with Anti-shake ON) |

| Drive | Drive modes: Single, Continuous, 10 seconds/2 seconds self-timer, Single bracket, Continuous bracket Continuous advance: Approximately 3 frames per second Approximate number of frames that can be captured at one time: 3008x2000; 2256x1496; 1504x1000. RAW; 9 frames; -; -. RAW+JPEG; 9 frames; 9 frames; 9 frames. Extra fine; 12 frames; 14 frames; 20 frames. Fine; 15 frames; 19 frames; 30 frames. Standard; 19 frames; 26 frames; 43 frames. Exposure bracketing: With 0.3/0.5 EV increments, 3/5 frames Interval recording: Interval time: 30 seconds/1-10, 15, 20, 30, 45, 60 minutes, 2 -240 frames. Start timer is equipped. (Setting: 0.5 to 24H, in 0.5 increments) Other functions: Instant playback, Customizing, Zone matching |
| --- | --- |
| Anti-Shake | CCD-Shift Anti-shake system with LED indicator in viewfinder Anti-shake effect: Equivalent to 2 – 3 steps in shutter speed (Actual effect varies depending on shooting conditions and the lens used.) |
| Viewfinder | Type: Eye-level fixed pentaprism Focusing screen: Spherical Acute Matte (G-type as standard) Field of view: Approximately 95% Magnification: 0.9x (with 50mm lens focused on infinity) Diopter control: –3.0 to + 1.0 m-1 Eye-relief: 25mm from the eyepiece, 21mm from the eyepiece frame in -1 m-1 Eyepiece cup is removable. Interchangeable focusing screen is available (type M, L, ML) at service facilities. |
| Video Output | NTSC / PAL |
| Audio Signal | Yes |
| Printing Output Control | Exif Print, PRINT Image MatchingⅡ, PictBridge |
| PC Interface | USB 2.0 High-Speed |
| Remote Function | Remote data storage: available. (Remote-storage function software 'DiMAGE Transfer' for Maxxum7D is available as a free download. It will be released in January 2005. |
| Power | Battery: One NP-400 Li-ion battery Battery condition indication: 4-stage indicator appears when the camera is turned on. External power source: 6V DC (with specified AC adapter AC-11), Vertical Control Grip VC-7D |
| Battery Performance (with Fresh Batteries) | Number of recorded images: - Approximately 400 frames. (CIPA measurement*1) Conforming to CIPA standard test method with a compact flash card and an NP-400 lithium-ion battery. - Approximately 600 frames (Konica Minolta measurement A*2) - Approximately 2000 frames (Konica Minolta measurement B*3) Conditions: Anti-Shake: ON, Recording media: 512MB CompactFlash Cards, Battery: Li-ion Battery NP-400, *1:CIPA: Camera & Imaging Products Association *2: Flash: OFF, other conditions are same as CIPA measurement. *3: Drive mode: 5 frames continuous shooting, Flash ;OFF, Instant Playback: NO |

| Dimensions (W x H x D) | 1 5.91 x 4.17 x 3.05 inches (150 x 106 x 77.5mm) |
| --- | --- |
| Weight | 26.81 ounces (760g) (without batteries and recording media) |
| Operating Temperature | 32-104 °F (0 - 40 °C) |
| Standard Accessories | DiMAGE Viewer CD-ROM, Li-ion Battery NP-400, Li-ion Battery Charger BC-400, Wide Strap WS-4, Shoe-Cap SC-1000, Eyepiece CapⅡ, Eyepiece Cup EC-1400, Body Cap BDC-1000, Remote Cord Clip, LCD Monitor Protect Panel MPP-1000, USB Cable USB-2, Video Cable VC-500 |
| Optional Accessories | Vertical Control Grip VC-7D, Camera Case CS-7D, AC Adapter AC-11, SD-CF Card Adapter SD-CF 1, Remote Cord RC-1000S, Remote Cord RC-1000L, DiMAGE Master Software |

Compatible computers: IBM PC/AT compatible computers:

Window 98*/Me/2000(Professional)/XP(Home/Professional)

Macintosh OS9.0-9.2.2/OS X v10.1.3-10.1.5/v10.2.1-10.2.8/v10.3-10.3.5

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

*	Users with Windows 98 and Windows 98 Second Edition operating system must install dedicated driver software included in the DiMAGE viewer CD-ROM.

MAXXUM, DiMAGE, CxProcess are trademarks or registered trademarks of Konica Minolta Camera, Inc.

Windows is either a registered trademark or a trademark of Microsoft Corporation in the United States and/or other countries. Macintosh is a trademark or registered trademark of Apple Computer Inc. Other corporate names and product names are trademarks or registered trademarks of their respective companies.

Specifications, accessories and system requirements are subject to change without notification.

#   #   #