Konica Minolta DiMAGE X50 Digital Camera

Specifications

| Number of effective pixels | 5.0 million |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CCD | 1/2.5-type interline primary-color CCD Total pixels: 5.4 million |  |  |  |  |  |
| Camera sensitivity | Automatic (between ISO 50 - 160 equivalent), ISO 50,100, 200, 400 |  |  |  |  |  |
| Focal length | 6.1 – 17.1mm (35mm equivalent: 37 – 105mm) |  |  |  |  |  |
| Lens construction | 11 elements in 9 groups |  |  |  |  |  |
| Maximum aperture | f/2.8 – f/5.0 |  |  |  |  |  |
| Minimum focus distance | 4 in. (0.10m) from front of camera 2.4 in. (0.06m) (Super Macro) |  |  |  |  |  |
| Maximum magnification | 0.1x (1:10) Equivalent to 0.82x (1: 1.22) in 35mm format |  |  |  |  |  |
| Area covered at maximum magnification | 1.63 x 1.25 in. (42.3 x 31.8mm) |  |  |  |  |  |
| Viewfinder type | Optical real-image zoom viewfinder |  |  |  |  |  |
| Monitor LCD | 2.0 inch (5.2cm) digital-interface TFT color monitor with anti-reflection coating Total pixels: Approximately 115,000 |  |  |  |  |  |
| Shutter | CCD electronic shutter and mechanical shutter |  |  |  |  |  |
| Shutter speed range | 4 - 1/1000 seconds |  |  |  |  |  |
| Autofocus system | Video AF |  |  |  |  |  |
| Autofocus areas | Wide focus area (five-point AF) Spot focus area |  |  |  |  |  |
| Metering | Multi-segment, Spot |  |  |  |  |  |
| Multi-segment metering | 256 segments |  |  |  |  |  |
| Exposure control | Programmed AE |  |  |  |  |  |
| Automatic Digital Subject Program Selection | Portrait, Sports action, Landscape, or Sunset is automatically selected (Manual selection is also possible) Night portrait/Night view, Super Macro, Text is set manually only. |  |  |  |  |  |
| Exposure compensation | ±2 EV in 1/3 increments |  |  |  |  |  |
| Flash range | Wide: Approximately 0.7 – 10.5 ft. (0.2 – 3.2m) Telephoto: Approximately 0.3 – 5.9 ft. (0.1 – 1.8m) (Camera sensitivity: auto) |  |  |  |  |  |
| Flash modes | Autoflash, Autoflash with red-eye reduction, Fill-flash, Flash cancel |  |  |  |  |  |
| Continuous advance | Available, Speed: Approximately 1.9 frames per second* |  |  |  |  |  |
| UHS (Ultra High Speed) | Available, Speed: Approximately 11 frames per second*(1280 x 960) * Actual speed may vary depending upon the subject and camera settings. |  |  |  |  |  |
| Self-timer | Approximately 10 seconds |  |  |  |  |  |
| Multi frame | To capture 9 sequential images in a 3x3 grid in a single frame |  |  |  |  |  |
| Image pasting | Embeds an image within another image |  |  |  |  |  |
| Movie recording | Recording time dependent on memory card capacity. Frame rate: 15 frames per second , 30 frames per second Night Movie, Movie frame capture, Movie clip capture |  |  |  |  |  |
| Audio | Audio recording: Maximum 180 minutes* *128MB MB memory card is required. Voice memo: Maximum 15 seconds Audio caption: Maximum 15 seconds Monaural audio only |  |  |  |  |  |
| White balance control | Automatic, Preset (Daylight, Cloudy, Tungsten, Fluorescent) |  |  |  |  |  |
| Color modes | Color, Black and white, Sepia |  |  |  |  |  |
| Noise reduction | User selectable |  |  |  |  |  |
| Digital zoom | Maximum 4.3 x Digital zoom can be disabled. |  |  |  |  |  |
| E-mail Copy function | Creates a compressed 640x480 JPEG image for transmission. |  |  |  |  |  |
| Crop frame | Trims parts of captured still pictures and save the image as a separate file. |  |  |  |  |  |
| Date imprinting | Year/month/day (Format can be changed.) Month/day/hour:minute (Format can be changed.) Imprinting can be disabled. |  |  |  |  |  |
| Image-quality modes | Fine, Standard, Economy |  |  |  |  |  |
| Number of recorded pixels (Still images) | 2560 x 1920, 2048 x 1536, 1600 x 1200, 640 x 480, 1280 x 960 (UHS Continuous advance) |  |  |  |  |  |
| Number of recorded pixels (Movie clips) | 320 x 240 |  |  |  |  |  |
| Storage capacity | Approximate storage capacity of a 16MB SD (Secure Digital) Memory Card: |  |  |  |  |  |
|  |  | 2560x1920 | 2048x1536 | 1600x1200 | 640x480 | 1280x960 (UHS) |
|  | Fine* | 5 frames | 8 frames | 14 frames | 68 frames | 21 frames |
|  | Standard* | 11 frames | 17 frames | 27 frames | 99 frames | 38 frames |
|  | Economy* | 21 frames | 31 frames | 47 frames | 148 frames | 68 frames |
|  | Movie (320x240) | 15 frames per second: 41 seconds 30 frames per second: 21 seconds |  |  |  |  |
|  | Audio recording | 30 minutes 28 seconds |  |  |  |  |
|  | * Still images recorded without voice memo or audio caption ** Actual number of images may vary depending upon the subject and memory card. |  |  |  |  |  |
| File formats | JPEG, Motion JPEG (MOV), WAVE DCF 1.0-compliant DPOF-compliant (Supported by printing functions in version 1.1) Exif 2.2 |  |  |  |  |  |

| Folder name formats | Standard, Date |
| --- | --- |
| Printing output control | Exif Print, PRINT Image Matching II |
| Menu languages | Japanese, English, German, French, Spanish, and Chinese (Simplified) |
| Recording media | SD (Secure Digital) Memory Cards, MultiMediaCards |
| Computer interface | USB Full-speed 12Mbps data transfer with USB 2.0 compatible computer |
| Direct print | PictBridge |
| Remote camera | Live images without audio viewed with a computer |
| Battery | One NP-700 lithium-ion battery |
| Battery performance (Playback) | Approximate continuous playback time: 170 minutes Based on Konica Minolta’s standard test method with NP-700 lithium-ion battery |
| Battery performance (Recording) | Approximate number of recorded images: 150 frames. Based on CIPA (Camera & Imaging Products Association) measurements, when using standard battery (Lithium-ion battery) and standard memory card (SD, 16MB) |
| External power source | 4.7V DC with specified AC adapter (sold separately) |
| Dimensions (WxHxD) | 3.28 x 2.44 x 0.93 inches (83.5 x 62 x 23.5mm) |
| Weight | Approximately 4.4 oz. (125 g) without battery or recording media |
| Operating temperature | 32-104°F (0-40ºC) |
| Standard accessories | Lithium-ion Battery NP-700, Lithium-ion Battery Charger BC-800, Hand Strap HS-DG130, USB Cable USB-3, 16MB SD (Secure Digital) Memory Card, DiMAGE Viewer CD-ROM |
| Optional accessories | AC Adapter set AC-4, Metal Chain Neck Strap NS-DG100, Deluxe Case for DiMAGE X Series, Casual case for DiMAGE X Series, Marine Case MC-DG400, O-Ring set OR-GS400 (Replacement for MC-DG-400) |

Konica Minolta DiMAGE X50 Marine Case

Specifications

| Maximum depth (underwater) | 131 ft. (40m) |
| --- | --- |
| Weight | 8.5 oz. (240g) (Case only) |
| Dimensions (WxHxD) | 4.82 x 3.52 x 2.13 in. (122.5 x 89.5 x 54mm) (excluding protrusions) |
| Standard accessories | Hand strap, O-ring grease , Desiccant |
| Optional accessories | O-ring set |

System Requirements

Compatible computers when the camera is used as mass storage devise:

IBM PC/AT compatible computers with Windows XP (Home/Professional), Windows Me, Windows 2000 Professional, Windows 98*, or Windows 98 Second Edition* and with a USB port as standard interface.

Apple Macintosh computers with preinstalled Mac OS 9.0 – 9.2.2, Mac OS X v10.1.3-10.1.5, v10.2.1-10.2.8, v10.3-10.3.3 and with a USB port as standard interface.

*	Users with Windows 98 and Windows 98 Second Edition operating systems must install dedicated driver software included in the DiMAGE X50 package.

Minimum system requirements for the remote camera function:

| Computers | IBM PC/AT compatible computers with a USB port as standard interface |
| --- | --- |
| CPU | Pentium II 300MHz or later processor (*1) |
| Operating systems | Windows XP (Home/Professional), Me, 2000 Professional, or 98 Second Edition |
| RAM | 128 MB of RAM. 256 MB with Windows XP. (*1) |
| Hard-disk space | Approximately 200 MB of free hard-disk space |
| Monitor | 800 x 600 monitor capable of displaying 16-bit color quality. |
| Tested applications | Microsoft NetMeeting, Windows Messenger (*2) |
| Other | CD-ROM drive |

*1	The CPU and RAM must satisfy the requirements of the operating system.

*2 	The application software must be guaranteed by the manufacturer to work with the operating system.

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

Specifications, accessories and system requirements are subject to change without notice.

#   #   #

DiMAGE, CxProcess are trademarks or registered trademarks of Konica Minolta Photo Imaging, Inc.

Windows is either a registered trademark or a trademark of Microsoft Corporation in the United States and/or other countries. Macintosh is a trademark or registered trademark of Apple Computer Inc. Other corporate names and product names are trademarks or registered trademarks of their respective companies.