Konica Minolta DiMAGE X1 8 Megapixel Digital Camera

Specifications

| Number of effective pixels | Approximately 8.0 million (**3296 x 2460) |
| --- | --- |
| CCD | 1/1.8-type interline primary-color CCD **Total pixels: approximately. 8.3 million (3320 x 2496) |
| Focal length | 7.7 - 23.1mm (equivalent to 37 - 111mm in 35mm format) |
| Max. aperture | f/3.5 - f/3.8 |
| Focusing range | Wide-angle: 3.9 inches (0.1m) to infinity, Telephoto: 1.3 feet (0.4m) to infinity (from front of camera) Maximum magnification: 0.12 x (equivalent to 0.56x in 35mm format) (Focal length: 13.0mm, equivalent to 62mm in 35mm format, shooting distance: 3.9 inches (10cm)) Area covered at maximum magnification: 2.4 x 1.8 inches (61 x 46mm) Super Macro: 2 inches (0.05m) to infinity (from front of camera) (Focal length: 13.0mm, equivalent to 62mm in 35mm format) Maximum magnification: 0.21x (equivalent to 1.02x in 35mm format) Area covered at maximum magnification: 1.3 x 1 inch (34 x 25mm) |
| Recording media | SD (Secure Digital) Memory Cards, MultiMediaCards |
| File format | Still Image: JPEG, DCF2.0-compliant, DPOF-compliant (supported by printing functions in version 1.1) (On/off selectable on both index print and date imprinting), Exif 2.2. Movie: Motion JPEG (MOV) with monaural audio. Voice: WAVE (monaural audio only) |
| Number of recorded pixels | 3264 x 2448, 2560 x 1920, 2048 x 1536, 1600 x 1200, 640 x 480 |
| Image quality modes | Fine, Standard, Economy |
| LCD monitor | 2.5-inch (6.3cm) low-temperature polysilicon TFT color, Total pixels: Approximately 118,000, Field of view: Approximately 100% |
| Autofocus system | Video AF |
| Autofocus area | Changeable between spot and wide (five-point AF) |
| Exposure mode | Programmed AE |
| Metering | Multi-segment (300 segments), Spot metering |
| Exposure compensation | +/-2 EV in 1/3 increments |
| Camera sensitivity | Automatic, ISO 50, 100, 200 equivalents. **Automatic: between ISO 50 - 160 equivalent. **When the flash fires, the automatic sensitivity gain is between ISO 50 - 160 equivalent. |
| Shutter | CCD electronic shutter and mechanical shutter |
| Shutter speed range | 1 - 1/1250 second |
| Flash metering | Pre-flash TTL |
| Guide number | Approximately 21 feet (6.5 in meters) at ISO 100 |
| White balance control | Automatic, Preset (Daylight, Cloudy, Tungsten, Fluorescent), Setting possible in movie recording mode (Same with still images) **Method:TTL |
| Continuous advance | Approximately 0.5 frames per second Number of frames: Approximately 3 frames (3264 x 2448, Standard) Progressive capture selectable. |
| Color modes / Color space | Color, B/W, and Sepia |
| Image correction | Contrast, Sharpness (3 levels) |
| Playback functions | Single-frame playback, Index playback (6 frames), Enlarged playback (6x (maximum) in 1.1x increments, scrollable), Movie, Audio track indicator, Camera status, E-mail Copy function, Crop frame, Slide show, Rotate (manual) |
| Printing output control | Exif Print, Print Image Matching III, PictBridge |
| Movie recording functions (format) | Night Movie. Focusing and optical zooming during recording. Each settings are selectable (ON/OFF) in the menu. Default: OFF. |
| Number of recorded pixels (movie) | 640 x 480, 320 x 240 |
| Frame rate | 20 frames per second |
| Movie recording time | Until the Memory Card is full (dependent on memory card capacity and battery power.) |
| Audio | Audio recording, Audio caption |
| Battery | One rechargeable lithium-ion battery |
| Battery performance (still) | Approximately 150 frames (Conforming to CIPA*1 (included Lithium-ion battery, included SD memory card)) *1CIPA: Camera & Imaging Products Association **CIPA test method: Image size: 3264 x 2448, standard image quality, no instant playback, Flash used with 50% of the frames, etc. |
| Battery performance (playback) | Approximately 140 minutes (Based on Konica Minolta's standard test method: with included lithium-ion battery) |
| PC interface | USB (with specified cradle)(Full-speed 12Mbps data transfer available with USB 2.0 compatible computer) ** High speed (480Mbps) is not supported. |
| AV output | NTSC/PAL (selectable on the camera)(with specified cradle) |
| Included accessories | Hand strap, AV cable, USB cable, battery charger, Lith-ion battery, soft camera case, 32MB Secure Digital (SD) card, DiMAGE Digital Camera software |
| Optional accessories | AC adapter, metal chain neck strap, Marine case |
| Supported operating system | Data storage: Windows 98/98 SE/Me/2000(Professional)/XP(Home/Professional), Mac OS 9.0-9.2.2/Mac OS X v10.1.3-10.1.5/v10.2.1-10.2.8/v10.3-10.3.9/v10.4-10.4.1 |
| Dimensions (WxHxD) | 3.7 x 2.7 x 0.8 inches (95 x 68 x 19.5 mm) (Except projection) |
| Weight | Approximately 4.8 ounces (135 g) without batteries or recording media |

Compatible Computers

• IBM compatible PCs with built-in USB port running Windows XP (Professional/Home Edition), Windows Me, Windows 2000 Professional, Windows 98, Windows 98 SE

• Apple Macintosh computers with built-in USB port running Mac OS 9.0-9.2.2, Mac OS X version 10.1.3-10.1.5, 10.2.1-10.2.8, 10.3-10.3.9, 10.4-10.4.1.

OS Compatible with DiMAGE Master Lite

| Windows | Macintosh |
| --- | --- |
| Windows 98, Windows 98 Second Edition, Windows Me, Windows 2000 Professional, Windows XP Home Edition, Windows XP Professional | Mac OS X v10.1.3-10.1.5, v10.2.1-10.2.8, v10.3-10.3.9, v10.4-10.4.1 |

OS Compatible with Kodak EasyShare

| Windows | Macintosh |
| --- | --- |
| Windows 98, Windows 98 Second Edition Windows Me, Windows 2000 Professional (Service Pack 1~) Windows XP Home Edition, Windows XP Professional | Mac OS X v10.2.3-10.2.8, v10.3-10.3.8 |

Specifications, system requirements and accessories are subject to change without notice.

#   #   #

The Konica Minolta logo and “The essentials of imaging” are trademarks or registered trademarks of Konica Minolta Holding, Inc. DiMAGE and CxProcess are trademarks or registered trademarks of Konica Minolta Photo Imaging, Inc. Windows is a registered trademark of Microsoft Corporation in the United States and other countries. Apple, Macintosh and Mac OS are trademarks of Apple Computer, Inc., registered in the U.S. and other countries. EasyShare is a trademark or a registered trademark of Eastman Kodak Company. All other brand and product names are trademarks or registered trademarks of their respective owners.