Konica Minolta 5 Megapixel DiMAGE A1 Digital Camera

Specifications

| Number of effective pixels | 5.0 million |
| --- | --- |
| CCD | 2/3-type progressive primary-color CCD Total pixels: 5.3 million |
| A/D conversion | 14 bit |
| Camera sensitivity | Auto, ISO 100, 200, 400, 800 equivalents |
| Focal length | 7.2 – 50.8mm (35 mm equivalent: 28 - 200 mm) |
| Lens construction | 16 elements in 13 groups (includes two AD glass elements and two aspheric elements) |
| Maximum aperture | f/2.8 – f/3.5 |
| Minimum focus distance | 1.6 ft. (0.5m) from CCD |
| Macro focus range | At 7.2mm: 11.8 – 23.6 in. (30 – 60 cm) from CCD 8.3 – 20 in. (21 – 51 cm) from front of lens At 42.7 – 50.8mm: 9.8 – 23.6 in. (25 – 60 cm) from CCD 5.1 – 19.7 in. (13 – 50 cm) from front of lens |
| Maximum magnification | 0.177x (1: 5.65) Equivalent to 0.7x (1: 1.43) in 35mm format |
| Area covered at maximum magnification | 1.97 x 1.46 in. (50 x 37 mm) |
| Optical zooming control | Manual zooming ring |
| Filter diameter | 49mm |
| Viewfinder type | EVF (Electronic viewfinder) Variable-position: 0-90º |
| Viewfinder LCD | 0.44 inch (11mm) TFT liquid-crystal microdisplay Total pixels: 235,000 pixels |
| LCD monitor | 1.8 inch (46mm) low-temperature polysilicon TFT color Variable-position: -20- 90º Total pixels: 118,000 |
| Automatic monitor amplification | Activates in low light. |
| Field of view | EVF: Approximately 100% External LCD monitor: Approximately 100% |
| Display-mode switch | Auto-display, Electronic-viewfinder display, External LCD-monitor display |
| Diopter control | – 5 ~ +2 m-1 |
| Eye relief | 22mm at -1 m-1 |
| Shutter | CCD electronic shutter and mechanical shutter |
| Shutter speed range | 30 - 1/16,000 second Bulb (maximum 30 seconds) |
| Autofocus system | Video AF |
| Autofocus areas | Wide focus area, Spot focus point Flex Focus Point control is available with the spot focus point. |
| Focus modes | Autofocus: Single-shot AF, Continuous AF, Full-time AF, Subject Tracking AF. Manual focus with 2x or 8x Flex Digital Magnifier (FDM) |
| Full-time AF | With the spot focus point or Flex Focus Point. Full-time AF can be disabled. |
| Subject Tracking AF | With continuous AF. Subject Tracking AF can be disabled. |
| Focus lock | By pressing shutter-release button partway down. |
| Subject lock | With Subject Tracking AF by pressing shutter-release button partway down. |
| Metering | Multi-segment, Center-weighted, Spot (center or with Flex Focus Point) |
| Multi-segment metering | 300 segments |
| Exposure control range | Programmed AE and aperture-priority modes: Wide: EV –1 – 20 Telephoto: EV –0.4 – 20.7 Shutter-priority and manual modes: Wide: EV – 2 – 21 Telephoto: EV – 1.4 – 21 |
| Exposure modes | Programmed AE (with program shift), Aperture priority, Shutter priority, Manual |
| Digital Subject Programs | Portrait, Sports action, Sunset, Night portrait |
| Exposure compensation | ± 2 EV in 1/3 increments |
| AE lock | With AE-lock button or by pressing the shutter-release button partway down |
| Flash control | ADI, Pre-flash TTL, Manual flash control |
| Flash range | Wide: Approximately 1.6 – 12.5 ft. (0.5 - 3.8m) Telephoto: Approximately 1.6 – 9.8 ft. (0.5 - 3m) (Camera sensitivity: auto) |
| Flash modes | Fill-flash, Red-eye reduction, Rear flash sync, Wireless/Remote flash |
| Flash recycling time | Approximately 5 seconds |
| Flash compensation | ± 2 EV in 1/3 increments |
| Flash-sync speed | All shutter speeds |
| External flashes | Auto-flash metering with Program/Maxxum Flash 2500 (D), 3600HS (D), 5600HS (D), Macro Twin Flash 2400,* Macro Ring Flash 1200.* *Macro Flash Controller is necessary. Non-dedicated off-camera flash units can be used using the camera’s flash-sync terminal. |
| Continuous advance | - High-speed continuous advance: Maximum 2.8 frames per second - Standard continuous advance: Maximum 2 frames per second *Actual speed may vary depending upon the subject and camera settings. Approximate number of frames that can be captured at one time: 2560x1920 2080x1560 1600x1200 640x480 RAW: 5 frames - - - TIFF: 3 frames 3 frames 3 frames 3 frames Extra Fine: 3 frames 3 frames 3 frames 3 frames Fine: 3 frames 3 frames 3 frames 3 frames Standard: 3 frames 3 frames 3 frames 3 frames *Actual number of images may vary depending upon the subject and camera settings. |
| Interval recording | 2 – 240 frames Interval time: 30 seconds, 1 – 10 minutes, 15 minutes, 20 minutes, 30 minutes, 45 minutes, 60 minutes Start time: 0-24 hour in 0.5 hour increments. |
| Exposure bracketing | Three-frame bracket in 0.5 and 0.3 EV increments |

| Digital Enhanced Bracketing | Three-frame contrast, color saturation, or filter bracket in fixed increments. |
| --- | --- |

| Self-timer | Approximately 10 seconds, 2 seconds |
| --- | --- |
| Movie | - Standard movie mode: Maximum 15 minutes at 24 frames per second with monaural audio - Night movie mode: Maximum 15 minutes at 24 frames per second with monaural audio - Automatic selection function between standard and night movie modes available. - Time-lapse movie mode: no audio and played back at 4 frames per second |
| Audio | Voice memo: Maximum 15 seconds monaural audio with still image |
| Color modes | Natural Color (sRGB), Vivid Color (sRGB), Adobe RGB, Embedded Adobe RGB, Black and white (neutral and toned), Solarization |
| Camera color spaces | sRGB, Adobe RGB |
| White-balance control | Automatic Preset (Daylight, Tungsten, Shade, Cloudy, Flash, and Fluorescent settings) with fine adjustments in seven levels. Custom (three settings) |
| Sharpness settings | Three levels (Soft, Normal, Hard) |
| Digital Effects Control | Color saturation, Contrast compensation, Filter |
| Noise reduction | User selectable dark noise reduction |
| Digital zoom | 2x with still images |
| Data imprinting | Year/month/day (Format can be changed.) Month/day/hour:minute Text Text and serial numbers Imprinting can be disabled. |
| Custom functions | Control dial customization. User-selectable function can be assigned to the function dial. |
| Image-quality modes | RAW, TIFF, Extra fine, Fine, Standard |
| Number of recorded (Still images): | 2560 x 1920, 2080 x 1560, 1600 x 1200, 640 x 480 |
| Number of recorded (Movie clips): | 320 x 240 in standard and night movie modes 640 x 480 in time-lapse movie mode |
| Storage capacity | Approximate storage capacity of a 16 MB CompactFlash card: 2560x1920 2080x1560 1600x1200 640x480 RAW: 2 frames - - - TIFF: 1 frames 1 frames 2 frames 15 frames Extra Fine: 2 frames 4 frames 7 frames 35 frames Fine: 5 frames 8 frames 14 frames 47 frames Standard: 10 frames 14 frames 23 frames 63 frames Movie: 19 seconds with 320x240 images *Still images recorded without voice memo. **Actual number of images may vary depending upon the subject and memory card used. |
| File formats | JPEG, TIFF, Motion JPEG (MOV), RAW, WAVE DCF 1.0-compliant DPOF-compliant (Supported by printing functions in version 1.1) Exif 2.2 |
| Printing output control | Exif Print, PRINT Image Matching II |

| Recording media | Type I and Type II CompactFlash cards IBM Microdrive (170MB, 340MB, 512MB, 1GB) |
| --- | --- |
| Computer interface | Full-speed 12Mbps USB |
| AV output | NTSC, PAL (selected on the camera) |

| Batteries | One NP-400 lithium-ion battery |
| --- | --- |
| Battery performance (Playback) | Approximate continuous playback time: 300 minutes with NP-400 lithium-ion battery, External LCD monitor on, EVF off. |
| Battery performance (Recording) | Approximate number of recorded images: 330 frames with NP-400 lithium-ion battery, EVF on, External LCD monitor off, Full-size images (2560 x 1920), Fine image quality, No instant playback, No voice memo, Flash used with 50% of the frames. |
| External power source | 6V DC with specified AC adapter (sold separately) |
| Dimensions (WxHxD) | 4.61 x 3.35 x 4.47 inches (117 x 85 x 113.5mm) |
| Weight | Approximately 19.8 oz. (560 g) without batteries or recording media |
| Operating temperature | 32-104 ºF (0-40 ºC) |
| Standard accessories | Neck Strap NS-DG4000, Lens Cap (49mm) LF-1249, Accessory Shoe Cap SC-10, AV Cable AVC-400, 16MB CompactFlash Card, USB Cable USB-500, DiMAGE Viewer, Lens Shade DLS-1, Lithium-ion Battery NP-400, Lithium-ion Battery Charger BC-400 |
| Optional accessories | AC Adapter AC-1L, Leather Neck Strap NS-DG1000, Holding Strap HS-2, External High Power Battery Pack Kit EBP-100, Remote Cord RC-1000S, Remote Cord RC-1000L, Step Up Adapter 49mm – 62mm, CompactFlash Adapter for SD Memory Cards SD-CF1, Battery Pack BP-400, DiMAGE Capture |

System Requirements

IBM PC/AT compatible computers with Windows XP Home, Windows XP Professional, Windows Me, Windows 2000 Professional, Windows 98*, or Windows 98 Second Edition* and with a USB port as standard interface.

Apple Macintosh computers with Mac OS 9.0 – 9.2.2, Mac OS X v10.1.3-10.1.5, or v10.2.1 – 10.2.5 and with a USB port as standard interface.

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

*	Users with Windows 98 and Windows 98 Second Edition must install dedicated driver software included in the DiMAGE A1 package.

Specifications, accessories and system requirements are subject to change without notice.

#  #  #

Konica Minolta and DiMAGE are trademarks or registered trademarks of Konica Minolta Holdings, Inc.  Windows is either a registered trademark or a trademark of Microsoft Corporation in the United States and/or other countries. Macintosh is a trademark or registered trademark of Apple Computer Inc. Other corporate names and product names are trademarks or registered trademarks of their respective companies.