##### Konica Minolta DiMAGE G500 5 Megapixel Digital Camera

##### Specifications

| Number of effective pixels | 5.0 million |
| --- | --- |
| CCD | 1/1.76-type interline primary-color CCD Total pixels: 5.36 million |
| Camera sensitivity | Auto, ISO 50, 100, 200, 400 equivalents |
| Focal length | 8 – 24mm (35mm equivalent: 39 - 117mm) |
| Lens construction | 7 elements in 6 groups |
| Maximum aperture | f/2.8 – f/4.9 |
| Minimum focus distance | Wide: 1.6 ft. (0.5m)from front of lens Telephoto: 2.6 ft. (0.8m) from front of lens |
| Minimum macro distance | Wide: 2.4 in. (6cm) from front of lens Telephoto: 19.7 in. (50cm) from front of lens |
| Viewfinder type | Optical real-image zoom viewfinder |
| Monitor LCD | 1.5 inch (3.8cm) TFT color monitor Total number of pixels: 117,000 |
| Shutter | CCD electronic shutter and mechanical shutter |
| Shutter speed range | Programmed AE: 1 - 1/2000 second Manual exposure mode: 15 - 1/1000 second |
| Autofocus system | Video AF |
| Metering | Center-weighted, Spot |
| Exposure modes | Programmed AE, manual exposure |
| Exposure compensation | ±1.5 EV in 0.3 increments |
| Flash range | Wide: Approximately 1.6 – 11.5 ft. (0.5 – 3.5m) Telephoto: Approximately 1.6 – 6.6 ft. (0.5 – 2.0m) |
| Flash/Shooting modes | Autoflash, Autoflash with red-eye reduction, Fill-in, Off, Slow-sync, Macro-photo, Distant view, Self-timer, Fixed focus |
| Continuous advance | Available |
| Self-timer | Approximately 10 seconds, 3 seconds |
| Movie recording | Recording time: Maximum 30 seconds with monaural audio Frame rate: 15frames per second |
| Audio recording | Maximum 30 seconds of audio can be recorded with and without an image. Audio can be recorded and attached to a playback image. Monaural audio only. |
| White-balance control | Automatic, Preset (Daylight, Cloudy, Tungsten, Fluorescent) |
| Contrast compensation | Five levels |
| Color saturation compensation | Five levels |
| Sharpness settings | Five levels |
| Color balance | Five levels for each RGB channel |
| Color modes | Color, Black and white, Sepia |
| Digital zoom | 2x, 3x |
| Copy function | # To copy images and audio between the camera memory and memory # card, or between the memory cards. |
| Resize | To VGA (640x480) from 2592x1944, 2048x1536, or 1600x1200 size images for transmission. To QVGA (320x240) from 2592x1944, 2048x1536, 1600x1200, or 640x480 for transmission. |
| Compression modes | Fine, Normal |
| Number of recorded pixels (Still images) | 2592 x 1944, 2048 x 1536, 1600 x 1200, 640 x 480 |
| Number of recorded pixels (Movie clips) | 320 x 240 |
| Storage capacity | Approximate storage capacity of a 16MB SD (Secure Digital) Memory Card: 2592x1944 2048x1536 1600x1200 640x480 Fine: 6 frames 12 frames 20 frames 70 frames Normal: 11 frames 18 frames 35 frames 140 frames Movie clip: Three 30-second clips * Movie clips recorded with audio. * Actual number of images may vary depending upon the subject and memory card. |
| File format | JPEG, Motion JPEG (AVI), WAVE DCF-compliant DPOF-compliant (Supported by printing functions in version 1.1) Exif 2.2 |
| Printing output control | Exif Print |
| Camera memory | 2MB of non-volatile flash memory |
| Recording media | SD (Secure Digital) Memory Cards, MultiMediaCards, Memory Stick |
| Computer interface | Full-speed 12Mbps USB |
| Battery | One NP-500 lithium-ion battery |
| Battery performance (Playback) | Approximate continuous playback time: 90 minutes |
| Battery performance (Recording) | Approximate number of recorded images: 100 frames with LCD monitor on and flash used with 50% of the frames. |
| External power source | 4.2 V DC with specified AC adapter (sold separately) |
| Dimensions (WxHxD) | 3.7 x 2.2 x 1.16 inches (94 x 56 x 29.5mm) |
| Weight | Approximately 7.1 oz. (200g) without batteries or recording media |
| Operating temperature | 32-122°F (0-50ºC) |
| Standard accessories | #### Hand strap, Neck Strap NS-DG130, USB cable USB-800, 16MB SD #### (Secure Digital) Memory Card, Lithium-ion Battery NP-500, #### Lithium-ion Battery Charger BC-500U, DiMAGE Viewer CD-ROM |
| Optional accessories | AC Adapter AC-8U |

Minimum System Requirements

IBM PC/AT compatibles:

Computers: IBM PC/AT compatible computers with pre-installed Windows XP Home, Windows XP,

Professional, Windows Me, Windows 2000 Professional, Windows 98*, or Windows 98 Second Edition*

and with a USB port as standard interface.

RAM: 16MB of RAM (32MB or more recommended)

Monitor: 640x480 monitor capable of displaying 32,000 colors

Others: CD-ROM drive

Apple Macintosh Computers:

Computers: Apple Macintosh computers with pre-installed Mac OS 9.0, 9.1, 9.2, and Mac OS X v.

10.0.4-10.2 and with a USB port as standard interface.

CPU: PowerPC

RAM: 16MB of RAM (32MB or more recommended)

Monitor: 640x480 monitor capable of displaying 32,000 colors

Others: CD-ROM drive

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

* Users with Windows 98 and Windows 98 Second Edition operating systems must install dedicated driver software included in the DiMAGE G500 package.

Specifications, accessories, and system requirements are subject to change without notice.

#   #   #

Konica Minolta and DiMAGE are trademarks or registered trademarks of Konica Minolta Holdings, Inc.  Windows is either a registered trademark or a trademark of Microsoft Corporation in the United States and/or other countries. Macintosh is a trademark or registered trademark of Apple Computer Inc. All other brand and product names are trademarks or registered trademarks of their respective owners.