Konica Minolta DiMAGE Z1 3.2 Megapixel Digital Camera

## Specifications

| Number of effective pixels | 3.2 million |
| --- | --- |
| CCD | 1/2.7-type interline primary-color CCD Total pixels: 3.3 million |
| Camera sensitivity | Auto and 50,100, 200, and 400 ISO equivalents |
| Focal length | 5.8 - 58mm (35mm equivalent: 38 – 380mm) |
| Lens construction | 10 elements in 7 groups (includes two elements with three aspheric surfaces) |
| Maximum aperture | f/2.8 – f/3.5 |
| Minimum focus distance | Wide: 1.6ft. (0.5m) from front of lens Telephoto: 6.6ft. (2m) from front of lens Wide: 1.9ft. (0.57m) from CCD Telephoto: 6.8ft. (2.07m) from CCD |
| Minimum macro distance | Super Macro Standard macro Focal length: 9.7mm* Wide Telephoto From front of lens: 1.6 in. (4 cm) 3.9 in. (10cm) 47.2 in. (120cm) From CCD: 4.3 in. (11cm) 6.7 in. (17cm) 50 in. (127cm) * at 9.7mm only |
| Maximum magnification | 0.12x (1: 8.33) Equivalent to 0.79x (1: 1.27) in 35mm format |
| Area covered at maximum magnification | 1.3 x 1.73 inches (33 x 44mm) |
| LCD monitor | 1.5 inch (3.8cm) low-temperature polysilicon TFT color with anti- reflection coating Total number of pixels: 113,000 Automatic monitor amplification |
| Field of view | Viewfinder: Approximately 98% LCD monitor: Approximately 100 % |
| Display-mode switch | Viewfinder display, LCD-monitor display Switch Finder system allows the monitor to be viewed directly or through the viewfinder. |
| Diopter control | – 3 ~ +1m-1 |
| Eye relief | 18.8mm |
| Shutter | CCD electronic shutter and mechanical shutter |
| Shutter speed range | 4 - 1/1000 second in programmed-AE and aperture-priority modes. 15 - 1/1000 second in shutter-priority and manual exposure modes. 2 - 1/1000 second in auto-recording and Digital Subject Programs Bulb (maximum 30 seconds) |
| Autofocus system | Rapid AF: a combination of a passive AF sensor with the video AF system |
| Autofocus areas | Wide focus area Spot focus areas with Focus Area Selection |
| Focus modes | Single-shot AF, Full-time AF, Manual focus. |
| Focus lock | By pressing shutter-release button partway down. |
| Metering | Multi-segment, Center-weighted, Spot |
| Multi-segment metering | 256 segments |
| Exposure modes | Programmed AE, Aperture priority, Shutter priority, Manual |
| Digital Subject Programs | Portrait, Sports action, Landscape, Sunset, Night portrait. Automatic and manual selection. |
| Exposure compensation | ±2 EV in 1/3 increments |
| AE lock | By pressing shutter-release button partway down |
| Flash metering | Pre-flash TTL |
| Flash range | Wide: Approximately 0.8 – 20 ft. (0.23 – 6.1 m) from CCD Telephoto: Approximately 3.9 – 15.7 ft. (1.2 – 4.8 m) from CCD (Camera sensitivity: auto) |
| Flash modes | Autoflash, Autoflash with red-eye reduction, Fill flash, Slow-shutter sync. Autoflash not available with aperture priority, shutter priority, and manual exposure modes. Flash cancel by pushing the built-in flash down. |
| Flash recycling time | Approximately 7 seconds |
| Flash compensation | ± 2 EV in 1/3 increments |
| Flash-sync speeds | All shutter speeds |
| External flashes | Auto-flash metering with Program/Maxxum Flash 2500 (D), Program/Maxxum 3600HS (D), Program/Maxxum Flash 5600HS (D) |
| Continuous advance | - UHS (Ultra High Speed) continuous advance: Approximately ten 1280x960 frames in one second - Standard continuous advance: Approximately 1.5 frames per second * Actual speed may vary depending upon the subject and camera settings. Approximate number of frames that can be captured at one time with the continuous-advance drive modes: UHS continuous advance (1280x960) Fine: 10 frames Standard: 10 frames Economy: 10 frames # Standard continuous advance 2048x1536 1600x1200 1280x960 640x480 Fine 5 frames 8 frames 13 frames 42 frames Standard 10 frames 16 frames 24 frames 67 frames Economy 19 frames 30 frames 42 frames 94 frames * Actual number of images may vary depending upon the subject and camera settings. |
| Progressive Capture | Maximum number of images that can be recorded with the Progressive Capture mode: UHS: 10 frames Standard: 6 frames |
| Exposure bracketing | Three frame bracket in 1.0, 0.5, 0.3 EV increments |
| Self-timer | Approximately 10 seconds |

| Movie | Recording time dependent on memory card capacity, image size, and frame rate. Frame rate: 15 frames per second, 30 frames per second Fast forward and rewind control. With and without monaural audio with playback volume control. Jogging Movie frame capture Night movie (automatic selection or disabled) |
| --- | --- |
| White balance control | Automatic, Preset (Daylight, Cloudy, Tungsten, Fluorescent, Flash), Custom |
| Digital Effects Control | Contrast compensation, Sharpness |

| Contrast compensation | Three levels (Low, Normal, High) |
| --- | --- |
| Sharpness settings | Three levels (Soft, Normal, Hard) |
| Color modes | Natural Color, Vivid Color, Black and White, Sepia |
| Noise reduction | User selectable dark noise reduction |
| Digital zoom | Maximum 4x in 0.1x increments |
| Copy function | To copy single or multiple images to the same or another memory card. |
| E-mail Copy function | Creates a compressed 640x480 or 160x120 JPEG image for transmission. |
| Date imprinting | Year/month/day (Format can be changed.) Imprinting can be disabled |
| Key customization | The function activated by the flash-mode button can be changed. |
| Image-quality modes | Fine, Standard, Economy |
| Number of recorded pixels (Still images) | 2048 x 1536, 1600 x 1200, 1280 x 960, 640 x 480 1280 x 960 in UHS continuous-advance mode |
| Number of recorded pixels (Movie clips) | 640x480, 320x240, 160x120 |
| Storage capacity | Approximate storage capacity of a 16MB SD (Secure Digital) Memory Card: # Still images 2048x1536 1600x1200 1280x960 640x480 Fine 9 frames 14 frames 22 frames 69 frames Standard 17 frames 27 frames 39 frames 100 frames Economy 32 frames 47 frames 69 frames 150 frames # Movie clips 640x480 320x240 160x120 30 fps 13 seconds 21 seconds 1 minutes 22 seconds 15 fps 26 seconds 41 seconds 2 minutes 30 seconds * Movie clips recorded with audio. * Actual number of images may vary depending upon the subject and memory card used. |
| File formats | JPEG, Motion JPEG (MOV) DCF 1.0-compliant DPOF-compliant (Supported by printing functions in version 1.1) Exif 2.2 |
| Folder name formats | Standard, Date |
| Printing output control | Exif Print, PRINT Image Matching II |
| Recording media | SD (Secure Digital) Memory Cards, MultiMediaCards |
| Computer interface | Full-speed 12Mbps USB |
| Direct print | USB DIRECT-PRINT |
| AV output | NTSC, PAL (selected on the camera) |
| Batteries | Four AA batteries: Alkaline, Ni-MH |
| Battery performance (Playback) | Approximate continuous playback time: 300 min. with alkaline batteries. |
| Battery performance (Recording) | Approximate number of recorded images: 250 frames with alkaline batteries, LCD monitor on, Full-size images (2048x1536), Standard image quality, No instant playback, No full-time AF, Flash used with 50% of the frames. |

| External power source | 6V DC with specified AC adapter (sold separately) |
| --- | --- |
| Dimensions (WxHxD) | 4.31 x 3.05 x 3.15 inches (109.5 x 77.5 x 80mm) |
| Weight | Approximately 10.8oz. (305g) without batteries or recording media |
| Operating temperature | 32-104°F (0-40ºC) |

| Standard accessories | Neck Strap NS-DG5000, Lens Cap LF-242, Accessory Shoe Cap SC-1000, AV Cable AVC-200, 16MB SD (Secure Digital) Memory Card, USB Cable USB-500, DiMAGE Viewer, ArcSoft VideoImpression CD-ROM, Four AA Alkaline batteries |
| --- | --- |
| Optional accessories | AC Adapter AC-1L, Leather Neck Strap NS-DG1000, External High Power Battery Pack Kit EBP-100, Wide Converter Kit ZCW-100 |

System Requirements

IBM PC/AT compatible computers with Windows XP Home, Windows XP Professional, Windows Me, Windows 2000 Professional, Windows 98*, or Windows 98 Second Edition* and with a USB port as standard interface.

Apple Macintosh computers with pre-installed Mac OS 9.0 – 9.2.2, Mac OS X v10.1.3 – 10.1.5, and v10.2.1 – 10.2.6 and with a USB port as standard interface.

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

*	Users with Windows 98 and Windows 98 Second Edition must install dedicated driver software included in the DiMAGE Z1 package.

Specifications, accessories and system requirements are subject to change without notice.

#   #   #