##### Konica Minolta DiMAGE G400 4 Megapixel

##### Digital Camera

##### Specifications

| Number of effective pixels | 4.0 million |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CCD | 1/2.5-type interline primary-color CCD Total pixels: 4.2 million |  |  |  |  |  |
| Camera sensitivity | Auto, ISO 50, 100, 200, 400 equivalents |  |  |  |  |  |
| Focal length | 5.6 – 16.8mm (35mm equivalent: 34 - 101mm) |  |  |  |  |  |
| Lens construction | 7 elements in 6 groups |  |  |  |  |  |
| Maximum aperture | f/2.8 – f/4.9 |  |  |  |  |  |
| Minimum focus distance | Wide: 1.6 ft. (0.5 m) from front of lens Telephoto: 2.6 ft. (0.8 m) from front of lens Wide: 1.8 ft. (0.54 m) from CCD Telephoto: 2.8 ft. (0.84 m) from CCD |  |  |  |  |  |
| Macro focus range |  |  | Super Macro | Standard macro |  |  |
|  | Focal length |  | Telephoto* | Wide |  | Telephoto |
|  | From front of lens |  | 7.9 – 15.7 in. (20 – 40 cm) | 2 in. to infinity (5cm) |  | 19.7 in. to infinity (50cm) |
|  | From CCD |  | 9.4 – 17.3 in. (24 – 44 cm) | 3.5 in. to infinity (9 cm) |  | 21.3 in. to infinity (54cm) |
|  | * at telephoto position only. |  |  |  |  |  |
| Area covered at maximum magnification | Approximately 3.1 x 2.4 in. (8 x 6 cm) |  |  |  |  |  |
| Viewfinder type | Optical real-image zoom viewfinder |  |  |  |  |  |
| LCD monitor | 1.5 inch (3.8cm) TFT color monitor Total number of pixels: 76,800 Automatic monitor amplification |  |  |  |  |  |
| Shutter | CCD electronic shutter and mechanical shutter |  |  |  |  |  |
| Shutter speed range | 1 - 1/2000 second in programmed AE and aperture-priority modes 15 - 1/1000 second in manual exposure mode |  |  |  |  |  |
| Autofocus system | Rapid AF: a combination of a passive AF sensor with the video AF system. |  |  |  |  |  |
| Metering | Center-weighted, Spot |  |  |  |  |  |
| Exposure modes | Programmed AE, Aperture priority, Manual |  |  |  |  |  |
| Exposure compensation | ±2 EV in 0.3 increments |  |  |  |  |  |
| Flash range | Wide: Approximately 1.6 – 6.9 ft. (0.5 – 2.1 m) Telephoto: Approximately 2.6 – 3.9 ft. (0.8 – 1.2 m) (Camera sensitivity: auto) |  |  |  |  |  |
| Flash modes | Autoflash, Autoflash with red-eye reduction, Fill-in, Fill-in with red- eye reduction, Off |  |  |  |  |  |
| Flash compensation | ± 1 EV in 0.5 increments |  |  |  |  |  |
| Scene selection | Portrait, Scenery, Night, Snap, Sports, Angel |  |  |  |  |  |
| Continuous advance | - Super Continuous: Approximately 2.5 frames per second - Standard continuous: Approximately 1.1 frames per second * Actual speed may vary depending upon the subject and camera settings. A maximum of 3 frames can be captured at one time with the Super Continuous drive mode. The number of frames with the standard-continuous drive mode is only limited by the memory card capacity. * Actual number of images may vary depending upon the subject and camera settings. |  |  |  |  |  |
| Auto bracketing | Exposure, Focus Exposure: Three-frame bracket in 0.3 EV increments Focus: Three-frame bracket |  |  |  |  |  |
| Self-timer | Approximately 10 seconds, 3 seconds |  |  |  |  |  |
| Movie recording | Recording time dependent on memory card capacity and image quality. Frame rate: 15 frames per second |  |  |  |  |  |
| Audio recording | Recording time dependent on memory card capacity. Monaural audio only. |  |  |  |  |  |
| White-balance control | Automatic, Preset (Daylight, Cloudy, Tungsten, Fluorescent) |  |  |  |  |  |
| Contrast compensation | Five levels |  |  |  |  |  |
| Color saturation compensation | Five levels |  |  |  |  |  |
| Sharpness settings | Five levels |  |  |  |  |  |
| Color balance | Five levels for each RGB channel |  |  |  |  |  |
| Color modes | Standard, Cold, Warm, Black and White, Sepia |  |  |  |  |  |
| Digital zoom | 2x |  |  |  |  |  |
| Resize | Creates a VGA (640 x 480) or QVGA (320 x 240) image for transmission. |  |  |  |  |  |
| Image quality modes | 4M Fine (2272 x 1704) 4M Normal (2272 x 1704) 2M (1600 x 1200) VGA (640 x 480) |  |  |  |  |  |
| Number of recorded pixels (Still images) | 2272 x 1704, 1600 x 1200, 640 x 480 |  |  |  |  |  |
| Number of recorded pixels (Movie clips) | 320 x 240 |  |  |  |  |  |
| Storage capacity | Approximate storage capacity of a 16 MB SD (Secure Digital) Memory Card: |  |  |  |  |  |
|  | 4M Fine | 4M Normal |  |  | 2M | VGA |
|  | 8 frames | 16 frames |  |  | 33 frames | 133 frames |
|  | * Movie clips recorded with audio. *Actual number of images may vary depending upon the subject and memory card. |  |  |  |  |  |
| File format | JPEG, Motion JPEG (AVI), WAVE DCF-compliant DPOF-compliant (Supported by printing functions in version 1.1) Exif 2.2 |  |  |  |  |  |
| Printing output control | Exif Print |  |  |  |  |  |
| Recording media | SD (Secure Digital) Memory Cards, MultiMedia Cards, Memory Stick |  |  |  |  |  |
| Computer interface | Full-speed 12Mbps USB |  |  |  |  |  |
| Battery | One NP-600 lithium-ion battery |  |  |  |  |  |

| Battery performance (Playback) | Approximate continuous playback time: 200 minutes |
| --- | --- |
| Battery performance (Recording) | Approximate number of recorded images: 185 frames with LCD monitor on, 4M Normal image quality (2272 x 1704) and flash used with 50% of the frames. |
| External power source | 4.7 V DC with specified AC adapter (sold separately) |
| Dimensions | Width: 3.68 in. / 93.5 mm Height: 2.19 in. / 55.5 mm Depth: from 0.91 in. (23 mm) to 1.02 in. (26 mm) at the LCD monitor |
| Weight | Approximately 5.1 oz. (145 g) without batteries or recording media |
| Operating temperature | 32-122°F (0-50ºC) |
| Standard accessories | #### Neck Strap NS-DG130, USB cable USB-810, 16 MB SD (Secure #### Digital) Memory Card, Lithium-ion Battery NP-600, Lithium-ion #### Battery Charger BC-600U, DiMAGE Viewer CD-ROM |
| Optional accessories | AC Adapter AC-9U |

Minimum System Requirements

IBM PC/AT compatibles:

Computers:	IBM PC/AT compatible computers with pre-installed Windows XP Home Edition, Windows XP Professional, Windows Me, Windows 2000 Professional, Windows 98*, or Windows 98 Second Edition* and with a USB port as standard interface.

RAM:	16MB of RAM (32MB or more recommended)

Monitor:	640x480 monitor capable of displaying 32,000 colors

Others:	CD-ROM drive

Apple Macintosh Computers:

Computers:	Apple Macintosh computers with Mac OS 9.0, 9.1, 9.2, and Mac OS X v. 10.0.4-10.2 and with a USB port as standard interface.

CPU:	PowerPC

RAM:	16MB of RAM (32MB or more recommended)

Monitor:	640x480 monitor capable of displaying 32,000 colors

Others:	CD-ROM drive

•	The computer and operating system must be guaranteed by their manufacturers to support USB interface.

•	Problems may be encountered depending on what other USB devices are being used in parallel with this product.

•	Only a built-in USB port is supported. Problems may be encountered when the camera is connected to a USB hub.

•	Normal operation may not be possible even when all the system requirements are met.

*	Users with Windows 98 and Windows 98 Second Edition operating systems must install dedicated driver software included in the DiMAGE G400 package.

Specifications, system requirements and accessories are subject to change without notice.

Konica Minolta and DiMAGE are trademarks or registered trademarks of Konica Minolta Holdings, Inc. Windows is either a registered trademark or a trademark of Microsoft Corporation in the United States and/or other countries. Macintosh is a trademark or registered trademark of Apple Computer Inc. All other brand and product names are trademarks or registered trademarks of their respective owners.

#   #   #